# O2Loader

Reference Manual

by

MrX

# edited by

radish28

O2Loader - © 2004 MrX

# Creating the Packet

- Go into the UPPACKER directory and extract O2packer.exe & uppload.elf to the O2LOADER directory.

- If you want USB support, you need to download USBD.IRX. See the readme_for_usb_support.txt file in the O2LOADER directory.

- Run the appropriate BAT file to create the PACKET.FSH file (if you want a different LOGO replace mrxstation.bmp with your own 384x64 NTSC or 344x71 PAL logo)

- Run pukklink/ps2link or naplink software and load the uppacket.elf application. Use TRIANGLE to upload the packet. Also, you can build a CDROM image using CDGENPS2. Put PACKET.FSH, uppacket.elf and SYSTEM.CNF files into CDGENPS2 and burn the image.

# Preparing a USB Device

- You need to format the USB device to FAT12/16 or FAT32. O2loader will use the first partition on the USB device.

- Copy the content of the PENDRIVE directory to the USB device, and put your PS2 applications in the /BOOT/ directory (each application must have a .ELF  extension)

- If you want, you can put  ISO files in the root directory of the USB Device (not .bin files!). You must create .ISO files using CDGENPS2 3.0, the LBA fix is unnecessary.

- So in the BOOT directory is: config.dat and elf applications. In the root folder: ISO's for CDROM emulation.

# Preparing a HDD Device

- You need the Network Adapter and a Hard Disk with APA partitions. To format the HD and to create partitions, you can use HDDFRMT.ELF tool. You can download this application from: http://www.dms3.com/dms3/files/chips/DMS3_Tools/  (thanks to DMS for this utility :D). The partitions created can be used from PGEN emulator, for example (it use the sign '+' for 'public' partitions)

- Launch o2loader.elf to create O2_BOOT partition (you need 128MB free in the hard disk). You can put .elf applications in root directory of this partition to launch applications or save options from HDD.

# How I can upload files to my HDD?

Well, several programs to do that exist: you can use Execftp to upload using the FTP protocol or PS2Menu. For example, it is possible to mount a directory from the USB Device to use it like a CDROM and copy the programs or other files to the HDD partition. You can download PS2Menu from http://ps2dev.org/kb.x?T=1009

# Running O2loader

To launch the application, reset the console and hold the L2 button to use O2 MODE. When the application starts, you may see a warning message or error. When the modules are loaded and the program is initialized, you can see a list.  If you only see 'Run ISO from USB' then there is no applications on the HDD or USB device (is the HDD or USB device disconnected?).

'Run ISO from USB' will launch the ISO files in the root directory of the USB device. The other selections are: the application to launch from USB  (listed in white) or HDD (listed in green) devices.

You can use these buttons:

CROSS: To launch the elf without emulation

CIRCLE: To launch the elf selecting a directory to emulate CDROM (it creates a ISO structure with a maximum of 64 files)

SQUARE: To refresh the list. (for example, if you plug in a new USB device)

TRIANGLE: Turn to ISO emulation from USB.

SELECT:  To select the keywords for the autolaunch option (you need to select L2 button to launch all  keywords in O2 os)

UP/DOWN: To change the selection.

All the changes are saved to the HDD or USB device if HDD device is off. It remembers the last elf launched and the last iso launched.

Fig 1

# Running an ISO from USB

Select the iso using UP/DOWN and press CROSS to launch the iso. Use TRIANGLE to exit to the elf selection.

Fig 2

# Selecting a Directory for CDROM Emulation

First you select one hdd partition or mass (USB Devices) using the UP/DOWN and CROSS buttons. Then you can see the files (cyan color and not selectable) or directories (with a yellow folder icon). Open directories with CROSS and press CROSS in the option 'Select this directory' to launch the elf selected with CDROM emulation of the files in this directory (remember, the limit is 64 files). Use TRIANGLE to exit to the elf selection.

Fig 3: Select the partition

Fig 4: Select the directory

# Select the Keywords

If you press SELECT on an elf, you can select one or more keywords for autolaunch mode. In this mode, you can launch from OSDSYS a program from USB or HDD when a CDROM or DVD is detected and one extension is recognized.

You can use UP/DOWN to move, CROSS to Enable/Disable a Keyword and TRIANGLE to exit. This selection is saved in HDD or Pendrive. When you select one Keyword, it is displayed in Yellow color. Remember you need all selection to L2 button in the OS.

Fig 5: Select the keyword

# Error Messages

"ERROR: MODULES FOR USB OR HDD NOT PRESENT!!!"

This message is displayed if USBD.IRX does not exist and/or Modules for HDD are not complete (fails when trying load a module, for example). The program stops.

"ERROR! PSTWO NEED MODULE USBD.IRX!!!"

This message is displayed if USBD.IRX does not exist and the model of the console is a PSTWO  (HDD is not supported in PSTWO model). The program stops.

"WARNING: MODULE USBD.IRX NOT PRESENT!!!"

This message is displayed if USBD.IRX does not exist but you can use a HDD device.

"WARNING: MODULES FOR HDD NOT PRESENT!"

This message is displayed if one or more modules used in HDD fail or is not present but you can use USB device. If you do not have Network Adapter, this message is displayed too.

"ERROR! HDD NOT PRESENT!!!"

This message is displayed if the modules for HDD are loaded, but HDD cannot be detected.

"ERROR! UNFORMATED DISK!!!"

This message is displayed if the modules for HDD are loaded, but HDD is unformated.

"CREATING O2_BOOT PARTITION..."

Well, this message is not a error. It is displayed when O2loader trys to create the O2_BOOT partition in the HDD.

"ERROR! FAIL TO CREATE O2_BOOT PARTITION!!!"

This message is displayed when the O2_BOOT partition cannot be created for some reason.

"Error Loading Application"

This message is displayed if loading an elf file fails. This might happen if you run one ISO and SYSTEM.CNF is incorrect or if the tray is open.

# CDROM IMAGE SPECIFICATIONS

ISO EMULATION

Max Size of ISO: 2GB

Max Len for Filenames: 256 ANSI Chars (UNICODE not supported)

Program recomended for ISO creation: CDGENPS2 3.0

Only supported from USB devices.

AUTO-ISO EMULATION FROM DIRECTORY

Max Size of Emulated Files: 2GB

Max Number of Files: 64

Sub-directory Support: No

Max Len for Filenames: 253 Chars (you must include the path for filename)

Len recommended for Filenames: <45 chars